| Signals Contractor |  |  |
| --- | --- | --- |
| Civil Contractor |  |  |
| NEW CONTROLLER? | Circle: NO or YES |  |
| Make & Type: | Make: VC: ELV: Yes/No |  |
| ##### Signal Group Size / No. of | 8; 12; 16; 32. |
| Detector Card Size / No. of | 8; 12; 16; 32. |
| Labels & Numbering |  |  |
| Cabinet Earthed |  |  |
| Wiring complete and tidy |  |  |
| Stickers |  |  |
| ## NEW POLES or LANTERNS? | ##### Circle NO or YES. If yes, enter pole no’s: |  |
| Correct Location |  |  |
| Collar, coating, Finial Cap |  |  |
| Pole Numbers Installed |  |  |
| Lanterns have correct aspects |  |  |
| Lantern alignment and visors |  |  |
| Push buttons correct height |  |  |
| Push button sound, indicator light |  |  |
| ##### NEW DETECTORS? | Circle NO or YES. If yes, enter det no’s: |  |
| Loops Positioned Correctly |  |  |
| Sealant Installed Cleanly |  |  |
| KJB surround, slack, wiring joints |  |  |
| Check working on detector board |  |  |
| NEW CHAMBERS? | Circle NO or YES |  |
| Correct Ducting as per design |  |  |
| Chambers Level, Duct connections |  |  |
| 1m Cable Slack |  |  |
| CIVIL WORKS? | Tick or N/A for each item |  |
| Road & Path Surface Condition |  |  |
| Pram Ramps incl drainage |  |  |
| Tactile Pavers |  |  |
| Grass Berms |  |  |
| Signage |  |  |
| ##### NEW SOFTWARE? | Circle NO or YES |  |
| Flashing Amber, Phases Call |  |  |
| Ped protection OK |  |  |
| Operation OK incl special logic |  |  |
| OTHER | Tick or N/A for each item |  |
| SCATS comms |  |  |
| Camera connected & Working |  |  |
| New documentation (As-built, CIS) |  |  |

| ## OPERATION TESTING | ##### MUST BE COMPLETED IF ANY SIGNAL GROUP WIRING HAS CHANGED, BEFORE SWITCH ON |  |
| --- | --- | --- |
| ## Testing required? | ##### Circle NO or YES |  |
| Test | When to test | Complete? |
| Flash test each Signal Group | Preferred test. Must be safe to do so, e.g. roads are quiet/night or site is under traffic management control |  |
| Watch Signal Group operation | If not safe to do a flash test. Each signal group must be observed/checked. Diamond signal groups (e.g. those in a D1/D2 phase or G1/G2 phase) must have each of those phases locked on via SCATS or detector board. For example: Lock on all peds and the TH+RT detectors for the approach and check the correct signal groups run in each diamond phase, i.e. it should be SG1+3 together, SG2+4 together, etc. |  |

## REMEDIAL LIST

| No | Fault: | Action: | Completed Date: |
| --- | --- | --- | --- |
| ## Approved for switch on? YES / NO ## If yes, has the site been commissioned? YES / NO. ## Traffic Signal Representative:. ## Sign: Date:. ## Signal Contractor:. ## Sign: Date:. HAND OVER TO RCA OPERATIONS WHEN SIGNED ABOVE. |